ESTABLISHMENT OF BASESTATION

TRIMBLE – R8 (Static Method )

I )  Step

Base Station Fixing on  Known SOI Control Point.

Set up a Tripod with tribrach and adaptor over a Permanent Known Control point.

And Align the horizontal and Vertical Bubble centre.

Twist on the  0.25 mt Range pole to the tribrach adapter centerpiece  and Lock the tribrach and adaptor.

Measure the Height from Ground Center to Receiver centre of bumper. ( In Mts)

Twist the R8 Base Receiver to the pole. And Power up the Receiver.( Green Button)

II) Step

Base Controller Setup

Press Green button for Power on the Controller.

| Start | :- | Trimble Access |
| --- | --- | --- |
| Press Button | :- | Survey Styles |
| Select Method | :- | Static |
| Select Option | :- | Base Option |
| Fallow the Base Controller Setups |  |  |
| Survey Type | :- | Fast Static |
| Logging Interval | :- | 1 S |
| Elevation Mask | :- | 10° |
| Logging Device | :- | Controller |
| Antenna | :- | R8 GNSS/SPS88X |
| Press Next Screen |  | ⅓ |
| Measured to | :- | Center of Bumper |
| Antenna Height | :- | m |
| Serial No | :- | 5126468628 |
| Press Next Screen |  | ⅔ |
| GPS L2C | :- | √ ( Yes) |
| GLONASS | :- | √ ( Yes) |
|  |  | 3/3 |
| Press | :- | Accept Button |
| Press | :- | Store Option |

Press Esc Button

Now Your in Setting Screen Close this screen.

Now Your in Trimble Access Screen.

III )  Step ( Create BaseJob and BaseFile )

Select General Survey Button

Select Jobs

Create New Job

| Job Name | :- | 8-Aug-13 |  |
| --- | --- | --- | --- |
| Template | :- | Default |  |
| Coord .Sys | :- | 1:00000000 |  |
|  |  | Press Here |  |
| Select from | :- | Library |  |
| System | :- | UTM |  |
| Zone | :- | 44 North | Before Going to survey check UTM zone ( 43 / 44 / 45 ) |
| Datum | :- | Wgs 1984 ( 7P) |  |
| Use Goid Model | :- | No |  |
| Use Datum Grid | :- | No |  |
| Press Next Screen |  | 1/2 | Ending Page 1 |
| Co ordinate | :- | Grid |  |
| Project Height | :- | 500 | Hyderabad surroundings it is nearly 500 m. |
|  |  | 2/2 | Ending Page 2 |
| Press | :- | Store Button |  |
| Press | :- | Accept Button |  |

Now you are in Present Job. ( Example :- 8-Aug-2013).

IV )  Collecting Base Point.

Select -------- Instruments----- GNSS Function------Base Mode.

Press--------- Escape

Press --- Measure-- Static- Start Base Receiver.

| Point Name | :- | Base-8 Aug | Surveyor option |
| --- | --- | --- | --- |
| Select |  | Keyin |  |
| Northing | :- | 1926162.274 | Known Value enter |
| Easting | :- | 230974.082 | Known Value enter |
| Height | :- | 471.212 | Known Value enter |
| Antenna Height | :- | 1.730 m | Ground center to Center of Bumper |
| Measure to | :- | Center of bumper |  |

Press - Store

Press- Start

Now Base is Started .

1

2

3

8

5

6

7

9

4

10

11

12

13

14

14

V) Enable the Gagan Correction Signal  :-

Press No of Satellites available :-

Press Options

Controller Display

| Elevation Mask | :- | 10° |
| --- | --- | --- |
| PDOP | :- | 6.0 |
| Enable WAAS/EGNOS | :- | √ ( Yes) |

Press---- Accept

Press--- Escape.

Now Base is Started .

ROVER Settings

Rover Fix at place where control point shall be Established.

I ) Set up a Tripod with tribrach and adaptor over a Control point shall be established.

And Align the horizontal and Vertical Bubble centre.

Twist on the  0.25 mt Range pole to the tribrach adapter centerpiece  and Lock the tribrach and adaptor.

Measure the Height from Ground Center to Receiver centre of bumper. ( In Mts)

Twist the R8 Rover Receiver to the pole. And Power up the Receiver.( Green Button)

| Start | :- | Trimble Access |
| --- | --- | --- |
| Press Button | :- | Survey Styles |
| Select Method | :- | Static |
| Select Option | :- | Rover Option |
| Press Next Screen |  | ⅓ Ending page 1 |
| Antenna Height | :- | m |
| Remaining No change default only |  |  |
| Press Next Screen |  | ⅔ Ending page 2 |
| Tracking L2e | :- | √ ( Yes) |
| GPS L2C | :- | √ ( Yes) |
| GLONASS | :- | √ ( Yes) |
|  |  | 3/3 Ending page 3 |
| Press | :- | Accept Button |
| Press | :- | Store Option |

Press Escape Button

Now Your in Setting Screen Close this screen.

Now Your in Trimble Access Screen.

II) Step ( Create RoverJob and RoverFile )

Select General Survey Button

Select Job

Create New Job

Press Store

Press Accept

III) Collecting Base Point.

A)

Select -------- Instruments----- GNSS Function------Rover Mode.

Press--------- Escape

B)

Press --- Measure-- Static- Measure Points.

C)

| Point Name | :- | R1 | Rover First Point |
| --- | --- | --- | --- |
| Antenna Height | :- | x.xxxx m | Ground center to Center of Bumper |
| Remaining Keep Default only. |  |  |  |
| Press Next Screen |  | 1/2 | Ending Page 1 |

D)

| Options |  | Fast Static Times ( Set) |  |
| --- | --- | --- | --- |
| 4SV | :- | 12H0m0s | Min Observation 12 Hours |
| 5 SV | :- | 12H0m0s |  |
| 6 + SV | :- | 12H0m0s |  |
| Auto Store |  |  | Surveyor Choice only |
| Press Next Screen |  | 2/2 | Ending Page 2 |

Press ---- Accept  and End

Press---- Measure.

Note :- After 11 Minutes “Store” Option Highlighted

Press--- Store ( If we Select Auto store After 11 Minutes Automatically Stores )

Now Surveyor Ready to move Next Collecting Point. And fallow the III C Step.

End